CONTENT

1.  Transmission / Control Specification	4

2.  Transmission control method and characters	5

3.  Transmission format and characters	6

3.1  Command format and characters	6

3.2  Successful responsive package format and character	7

3.3  Failed responsive package format and character	7

4.  Address according to Multi-Device communication	8

5.  Communication Method	9

5.1  Ordinary operation	9

5.2  Irregular operation and back-up	9

6.  Card Status Code（st0,st1,st2）	13

7.  e1,e0 Error Code Table	14

8.  Command Specification	15

8.1  Reset (Initialization)	15

8.2  Status Request Command	16

8.3  Carry Card Command:	17

8.4  Entry Command	18

8.5  Auto-Check ICCard/RFCard Type:	19

8.5.1  Auto-Check ICCard Type:	19

8.5.2  Auto-Check RF Card Type:	20

8.6  CPUCard Operation	21

8.6.1  CPUCard Reset	21

8.6.2  Deactivate CPU Command	22

8.6.3  Inquire CPU Card Status:	22

8.6.4  CPU Card Communication T=0	23

8.6.5  CPU Card Communication T=1	24

8.6.7	CPU Warm Reset	25

8.6.7	T=1, T=0 CPU Card Protocol Automatic Communication	25

8.7	SAM(Secure Application Module) Control Command	26

8.7.1	Active SAM Command	26

8.7.2	Deactivate SAM Command	26

8.7.3	Inquire SAM Status Command	27

8.7.4	SAM Communication T=0	28

8.7.5	SAM Communication T=1	29

8.7.6	SAM Warm Reset	30

8.7.7	Auto-Check SAM Card T=0/T=1 Protocol	30

8.7.8	Select SAM	31

8.8 	SLE4442/4428 Control	32

8.8.1  SLE4442/4428 Reset	32

8.8.2  Deactivate SLE4442/4428	32

8.8.3  Inquire status of SLE4442/4428	33

8.8.4  SLE4442 Control	34

8.8.4.1  Data read from main memory on SLE4442	34

8.8.4.2  Data read from protection memory on SLE4442	35

8.8.4.3  Data read from security memory on SLE4442	36

8.8.4.4  Data write to main memory on SLE4442	36

8.8.4.5 	Data write to protection memory on SLE4442	37

8.8.4.5 	Data write to security memory on SLE4442	38

8.8.4.6 	Verification data present to SLE4428	39

8.8.5  SLE4428 Control	40

8.8.5.1.  Data Reading of main-memory of SLE4428	40

8.8.5.2.  Reading of protection-bit of SLE4428	41

8.8.5.3  Data writing to main-memory of SLE4428	42

8.8.5.4  Data writing to main-memory of SLE4428 with protecting	42

8.8.5.5  Written with protection-bit	43

8.8.5.6  Verification of password present to SLE4428	43

8.9  I2C Memory Card Control Command	44

8.9.1  Activate I2C memory card	44

8.9.2 	Deactivate I2C memory card	45

8.9.3	Inquire Status of I2C memory card	46

8.9.4  I2C Control	47

8.9.4.1  Read data from I2C	48

8.9.4.2  Write data to I2C	48

8.10  Contactless IC card Operation	49

8.10.1  Activated contackless IC card	49

8.10.1	Deactivate RFID card	51

8.10.2	Inquire status of RFID card	51

8.10.3	Mifare 1 card control	52

8.10.4	Key verification	52

8.10.4.2  Verify key from EEPROM	53

8.10.4.3  Modify sector key (KEY A)	54

8.10.4.4	  Download password to EEPROM	55

8.10.4.4  Read sector data	56

8.10.4.5  Write sector data	57

8.10.4.6  Initialization	58

8.10.4.7  Read value	59

8.10.4.8  Increment	60

8.10.4.9  Decrement	61

8.10.3  Type A RF card communication	62

8.10.4  Type B RFcard communication	63

8.11  Read Serial Number of CRT-571	64

8.11.1  Read serial number	64

8.11.2  Write Serial Number of CRT-571	64

8.11.3  Read CRT-571 configuration	64

8.11.3  Read CRT-571 configuration	65

8.11.4  Read CRT-571 version information	66

8.12.1  Error-card Bin Counter Control	66

8.13.1  Read error-card bin counter	66

8.13.2  Set initial value of error-card bin	67

Revision Log

| Version | Revision Time | Content |
| --- | --- | --- |
| 1.0 | 2010.01.10 | First Release |

- 1. Transmission / Control Specification

1) Baud rate: 9600/19200/38400/57600 BPS

2) Transmission method: Half duplex, Support multi-unit communication (16 units max)

3) Synchronous method: Asynchronous

| Start bit | D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 | Stop sbit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Start bit: 1 bit

Data length: 8bit

Check bit: none

Stop bit:1 bit

Character Code: ASCII 8 bit code

- 2 Transmission control method and characters

CRT-571 executes particular operation according to text (command) received from HOST then reports result of execution to HOST..

Character reference

ACK (06H)                      Acknowledge

NAK (15H)                      Negative acknow

EOT (04H)                      Clear the line

- 3 Transmission format and characters

- 3.1 Command format and characters

| STX (0xF2) | ADDR (1 byte) | LENH (1 byte) | LENL (1 byte) | CMT (1 byte) | CM (1 byte) | PM (1 byte) | DATA (N bytes) | ETX (1 byte) | BCC (1 byte) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Range of TEXT

Range of BCC calculation

MAX 1024 byte

| Code | Meaning |
| --- | --- |
| STX (F2H) | Representing the start of text in a command or a response. |
| ADDR | Representing the address of CRT-571 |
| LENH(1 byte) | Length of high byte |
| LENL(1 byte) | Length of low byte |
| CMT | Command head (‘C’ , 43H ) |
| CM | Specify as command. |
| PM | Command parameter |
| DATA | Transmission data ( N byte, N=0~512) |
| ETX (03H) | End of text |
| BCC | CRC Parity |

- 3.2 Successful responsive package format and character

| STX (0xF2) | ADDR (1 byte) | LENH (1 byte) | LENL (1 byte) | PMT (1 byte) | CM (1 byte) | PM (1 byte) | st0 (1 byte) | st1 (1 byte) | st2 (1 byte) | DATA (N bytes) | ETX (1 byte) | BCC (1 byte) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Range of TEXT

Range of BCC calculation

MAX 1024 byte

| Code | Meaning |
| --- | --- |
| STX (F2H) | Representing the start of text in a command or a response. |
| ADDR | Representing the address of CRT-571 |
| LENH(1 byte) | Length of high byte |
| LENL(1 byte) | Length of low byte |
| PMT | Return command head ( ‘P’ , 50H ) |
| CM | Specify as command. |
| PM | Command parameter |
| st0,st1,st2 | Status code |
| DATA | Transmission data ( N byte, N=0~512 ) |
| ETX (03H) | End of text |
| BCC (1 byte) | CRC Parity |

- 3.3 Failed responsive package format and character

| STX (0xF2) | ADDR (1 byte) | LENH (1 byte) | LENL (1 byte) | EMT (1 byte) | CM (1 byte) | PM (1 byte) | e1 (1 byte) | e0 (1 byte) | DATA (N bytes) | ETX (1 byte) | BCC (1 byte) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Range of TEXT

Range of  BCC calculation

MAX 1024 byte

| Code | Meaning |
| --- | --- |
| STX (F2H) | Representing the start of text in a command or a response. |
| ADDR | Representing the address of CRT-571 |
| LENH(1 byte) | Length of high byte |
| LENL(1 byte) | Length of low byte |
| EMT | Return command head ( ‘N’, 45H ) |
| CM | Specify as command. |
| e1,e0 | Status code |
| PM | Command parameter |
| DATA | Transmission data (N byte, N=0~512 ) |
| ETX (03H) | End of text |
| BCC (1 byte) | CRC Parity |

- 4. Address according to Multi-Device communication

ADDR：Address word for each device in multi-device communication

| Address | Character |
| --- | --- |
| 0# | 00H |
| 1# | 01H |
| 2# | 02H |
| 3# | 03H |
| 4# | 04H |
| 5# | 05H |
| 6# | 06H |
| 7# | 07H |
| 8# | 08H |
| 9# | 09H |
| 10# | 0AH |
| 11# | 0BH |
| 12# | 0CH |
| 13# | 0DH |
| 14# | 0EH |
| 15# | 0FH |

Notes:

Default address is 15#. Each device has unique address.

- 5. Communication Method

- 5.1 Ordinary operation

Command

ACK

(HOST)

(Execution)

ACK

Response

(ICRW)  bytes

- 5.2 Irregular operation and back-up

Command

ACK

ACK

Response

(HOST)

(ICRW)  bytes

(Execution)

Command

300msec Timeout

X error

Case 1

Case 2

(HOST)

(ICRW)  bytes

Command

ACK

Command

(Execution)

X error

ACK

Response

NAK

Case 3

Command

ACK

(HOST)

(ICRW)  bytes

NAK

(Execution)

Response

Response

ACK

Case 4

(HOST)

(ICRW)  bytes

20msec Timeout

Case 4

D

20sec Time out (Except Entry, Monitoring for removal,

Initialize, Intake/Withdraw command)

ACK

Command

Command

(Execution)

ACK

Response

ACK

Case 5

Command

(HOST)

(ICRW)  bytes

EOT

(Execution)

ACK

EOT

(discontinue)

Case 6

(HOST)

(ICRW)  bytes

300msec Timeout

Command

EOT

EOT

X error

(Execution)

ACK

EOT

(discontinue)

Case 7

Command

(HOST)

(ICRW)  bytes

EOT

ACK

X error

(Execution)

ACK

Response

| Chapter | Command | Function | CM | PM | description |
| --- | --- | --- | --- | --- | --- |
| 9.1 | INITIALIZE | Initialize CRT-571 | 30H | 30H | If card is inside, move card to cardholding position |
|  |  |  |  | 31H | If card is inside, capture card error card bin |
|  |  |  |  | 33H | If card is inside, does not move the card. |
|  |  |  |  | 34H | Same as 30H and retract counter will work. |
|  |  |  |  | 35H | Same as 31H and retract counter will work. |
|  |  |  |  | 37H | Same as 33H and retract counter will work. |
| 9.2 | STATUS REQUEST | Inquire status | 31H | 30H | Report CRT-571 status |
|  |  |  |  | 31H | Report sensor status |
| 9.3 | CARD MOVE | Card movement | 32H | 30H | Move card to card holding positon |
|  |  |  |  | 31H | Move card to IC card position |
|  |  |  |  | 32H | Move card to RF card position |
|  |  |  |  | 33H | Move card to error card bin |
|  |  |  |  | 39H | Move card to gate |
| 9.4 | CARD ENTRY | From output gate | 33H | 30H | Enable card entry from output gate |
|  |  |  |  | 31H | Disable card entry from ouput gate |
| 9.5 | CARD TYPE | ICCard/RFCard TypeCheck | 50H | 30H | Autocheck ICCardType |
|  |  |  |  | 31H | Autocheck RFCardType |
| 9.6 | CPUCARD CONTROL | CPU Card Applicatio Opertion | 51H | 30H | CPUCard cold reset |
|  |  |  |  | 31H | CPUCard power down |
|  |  |  |  | 32H | CPUCard status check |
|  |  |  |  | 33H | T=0 CPUCard APDU data exchange |
|  |  |  |  | 34H | T=1 CPUCard APDU data exchange |
|  |  |  |  | 38H | CPUCard hot reset |
|  |  |  |  | 39H | Auto distinguish T=0/T=1 CPUCard APDU data exchange |
| 9.7 | SAM CARD CONTROL | SAMCard Application Operation | 52H | 30H | SAMCard cold reset |
|  |  |  |  | 31H | SAMCard down power |
|  |  |  |  | 32H | SAMCard status check |
|  |  |  |  | 33H | T=0 SAMCard APDU data exchange |
|  |  |  |  | 34H | T=1 SAMCardAPDU data exchange |
|  |  |  |  | 38H | SAMCard hot reset |
|  |  |  |  | 39H | Auto distinguish T=0/T=1 SAMCardAPDU data exchange |
|  |  |  |  | 40H | Choose SAMCard stand |
| 9.8 | SLE4442/4428CARD CONTROL |  | 53H | 30H | SLE4442/4428Card reset |
|  |  |  |  | 31H | SLE4442/4428Card power down |
|  |  |  |  | 32H | Browse SLE4442/4428Card status |
|  |  |  |  | 33H | Operate SLE4442Card |
|  |  |  |  | 34H | Operate SLE4428Card |
| 9.9 | IIC MEMORYCARD | 24C01—24C256Card Operation | 54H | 30H | IICCard reset |
|  |  |  |  | 31H | IICCard down power |
|  |  |  |  | 32H | Check IICCard status |
|  |  |  |  | 33H | Read IICCard |
|  |  |  |  | 34H | Write IICCard |
| 9.10 | RFCARD CONTROL (13.56 MHZ) | Mifare standard card Type A & B T=CL protocol operation | 60H | 30H | RF Card startup |
|  |  |  |  | 31H | RF Card down power |
|  |  |  |  | 32H | RF Card operation status check |
|  |  |  |  | 33H | Mifare standard Card read/write |
|  |  |  |  | 34H | Type A standard T=CLCard APDU data exchange |
|  |  |  |  | 35H | Type B standard T=CLCard APDU data exchange |
|  |  |  |  | 39H | RF Card enable/disable |
| 9.11 | Card SERIAL NUMBER |  | A2H | 30H | Read Card Serial number |
| 9.12 | Read CARD CONFIG |  | A3H | 30H | Read Card configuration information |
| 9.13 | READ CRT-571 VERSION |  | A4H | 30H | Read Card software version information |
| 9.14 | RECYCLEBIN COUNTER |  | A5H | 30H | Read number of counter of Card error card bin |
|  |  |  |  | 31H | Initiate card error card bin counter |

- 6. Card Status Code（st0,st1,st2）

| st0 | Content |
| --- | --- |
| “0” | No Card in CRT-571 |
| “1” | One Card in gate |
| “2” | One Card on RF/IC Card Position |

| st1 | Content |
| --- | --- |
| “0” | No Card in stacker |
| “1” | Few Card in stacker |
| “2” | Enough Cards in card box |

| st2 | Content |
| --- | --- |
| “0” | Error card bin not full |
| “1” | Error card bin full |

- 7. e1,e0 Error Code Table

| e1,e0 | Content |
| --- | --- |
| “00” | Reception of Undefined Command |
| “01” | Command Parameter Error |
| “02” | Command Sequence Error |
| “03” | Out of Hardware Support Command |
| “04” | Command Data Error |
| “05” | IC Card Contact Not Release |
| “06”--“09” |  |
| “10” | Card Jam |
| “11” |  |
| “12” | sensor error |
| “13” | Too Long-Card |
| “14” | Too Short-Card |
| “15” |  |
| “16” | Card move manually |
| “17”--“39” |  |
| “40” | Move card when recycling |
| “41” | Magnent of IC Card Error |
| “42” |  |
| “43” | Disable To Move Card To IC Card Position |
| “44” |  |
| “45” | Manually Move Card |
| “46” |  |
| “47” |  |
| “48” |  |
| “49” |  |
| “50” | Received Card Counter Overflow |
| “51” | Motor error |
| “52”--“59” |  |
| “60” | Short Circuit of IC Card Supply Power |
| “61” | Activiation of Card False |
| “62” | Command Out Of IC Card Support |
| “63” |  |
| “64” |  |
| “65” | Disablity of IC Card |
| “66” | Command Out Of IC Current Card Support |
| “67” | IC Card Transmittion Error |
| “68” | IC Card Transmittion Overtime |
| “69” | CPU/SAM Non-Compliance To EMV Standard |
| “A0” | Empty-Stacker |
| “A1” | Full-Stacker |
| “A2”--“A9” |  |
| “B0” | Not Reset |

- 8. Command Specification

- 8.1 Reset (Initialization)

Command (TXET):

| “C” | 30H | Pm |
| --- | --- | --- |

Positive response (TXET):

| “P” | 30H | Pm | st0 | st1 | st2 | Rev_type |
| --- | --- | --- | --- | --- | --- | --- |

Negative response (TEXT):

| “N” | 30H | Pm | e1 | e0 |
| --- | --- | --- | --- | --- |

This is to set the operation conditions for SCT and to initialize CRT-571.

When it execute at first time, ICRW will auto check and judge HOST BAUD Rate.

CRT-571 is initialized in Disable mode that card is not accepted by this command.

CRT-571 is in prohibed status and return software version information.

Pm: Command parameter

If there is no card in CRT-571, engine will rotate slightly to clear up card in stacker.

If there are cards in CRT-571, the disposal is show as below:

30H: Move the card to Gate portion

31H: Capture card to reject-stacker

33H: If card is inside CRT-571, does not move the card.

34H: Same as pm=30H, and Retract counter will work.

35H: Same as pm=31H, and Retract counter will work

37H: Same as pm=33H, and Retract counter will work

Rev_type: softeware version, “CRT-571-V1.00”

- 8.2 Status Request Command

Command

| “C” | 31H | Pm |
| --- | --- | --- |

Positive response

| “P” | 31H | Pm | st0 | st1 | st2 | Sensor(10 byte) |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 31H | Pm | e1 | e0 |
| --- | --- | --- | --- | --- |

Pm=30H: Report current status of st0, st1, st2.

Pm=31H: Response is returned in form of Sensor, with the status information obtained.

The locations of sensor are referred to appearance drawing.

| Sensor | status |
| --- | --- |
| S1 | 30H No Card |
|  | 31H Have Card |
| S2 | 30H No Card |
|  | 31H Have Card |
| S3 | 30H No Card |
|  | 31H Have Card |
| S4 | 30H No Card |
|  | 31H Have Card |
| S5（reserved） |  |
| S6 | 30H No Card |
|  | 31H Have Card |
| S7 | 30H No Card |
|  | 31H Have Card |
| S8 | 30H No Card |
|  | 31H Have Card |
| S9 | 30H No Card |
|  | 31H Have Card |
| S10 | 30H No Card |
|  | 31H Have Card |

- 8.3 Carry Card Command:

Command

| “C” | 32H | Pm |
| --- | --- | --- |

Positive response

| “P” | 32H | Pm | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 32H | Pm | e1 | e0 |
| --- | --- | --- | --- | --- |

Pm=30H  	Move card to holding card position

Pm=31H 	Move card to IC Card position

Pm=32H	    Move card to RF Card position

Pm=33H	    Capture card to error card bin

Pm=39H	    Move card to the gate

If card can not move to specified position, CRT-571 will return Card jam error

Note: When execute Capture card command, if error card bin is full, CRT-571 will return error card bin error.

- 8.4 Entry Command

Command

| “C” | 33H | Pm |
| --- | --- | --- |

Positive response

| “P” | 33H | Pm | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 33H | Pm | e1 | e0 |
| --- | --- | --- | --- | --- |

Afert set card input from gate available, if insert card from gate, CRT-571 will carry the card to RF Card operation position.

Pm=30H	Enable card input from gate

Pm=31H	Disable card input from gate

Note: Execute reset command, CRT-571will disable card input from gate

- 8.5 Auto-Check ICCard/RFCard Type:

- 8.5.1 Auto-Check ICCard Type:

Command

| “C” | 50H | 30H |
| --- | --- | --- |

Positive response

| “P” | 50H | 30H | st0 | st1 | st2 | Card_type |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 50H | 30H | e1 | e0 |
| --- | --- | --- | --- | --- |

Auto-Check type of IC Card, and carry card to IC Card position，Auto Check IC Card Type,

Return Card_type information.

| Cart_type(2 byte) |  | Specification |
| --- | --- | --- |
| ‘0’ | ‘0’ | Unknown IC Card Type |
| ‘1’ | ‘0’ | T=0 CPU Card |
|  | ‘1’ | T=1 CPU Card |
| ‘2’ | ‘0’ | SLE4442 Card |
|  | ‘1’ | SLE4428 Card |
| ‘3’ | ‘0’ | AT24C01 Card |
|  | ‘1’ | AT24C02 Card |
|  | ‘2’ | AT24C04 Card |
|  | ‘3’ | AT24C08 Card |
|  | ‘4’ | AT24C16 Card |
|  | ‘5’ | AT24C32 Card |
|  | ‘6’ | AT24C64 Card |
|  | ‘7’ | AT24C128 Card |
|  | ‘8’ | AT24C256 Card |

- 8.5.2 Auto-Check RF Card Type:

Command

| “C” | 50H | 31H |
| --- | --- | --- |

Positive response

| “P” | 50H | 31H | st0 | st1 | st2 | Card_type |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 50H | 31H | e1 | e0 |
| --- | --- | --- | --- | --- |

Auto-Check type of RF card, carry card to RF card position, Auto-Check RF Card Type, Return Card_type information

| Cart_type(2 byte) |  | Specification |
| --- | --- | --- |
| ‘0’ | ‘0’ | UnknowRFCardType |
| ‘1’ | ‘0’ | Mifare one S50Card |
|  | ‘1’ | Mifare one S70Card |
|  | ‘2’ | Mifare one ULCard |
| ‘2’ | ‘0’ | Type A CPUCard |
| ‘3’ | ‘0’ | Type B CPUCard |

- 8.6 CPUCard Operation

- 8.6.1 CPUCard Reset

Command

| “C” | 51H | 30H | Vcc |
| --- | --- | --- | --- |

Positive response

| “P” | 51H | 30H | st0 | st1 | st2 | Type | ATR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 51H | 30H | e1 | e0 | ATR |
| --- | --- | --- | --- | --- | --- |

To cold reset IC card. The ICRW supplies power (VCC) and clock (CLK), return ATR.

Vcc=30H: CRT-571 supplies with +5V to VCC and activates in line with the EMV2000 ver4.0.

Vcc=33H: CRT-571 supplies with +5V to VCC and activates in line with the ISO/IEC7816-3.

Vcc=35H: CRT-571 supplies with +3V to VCC and activates in line with the ISO/IEC7816-3.

In case there is no Vcc word, it will have 30H as default value.

If ATR is not compliance to EMV, return e1,e0=“69”

Notes: There will be error and return ATR & Type when reset in line with EMV return.

If the IC card power error during reset, return e1, e0=”60”

Type: CPU Card protocol Type

=30H   T=0 protocol CPUCard

=31H   T=1 protocol CPUCard

Formate of ATR

| TS | TO | TA1 | TB1 | … | TCK |
| --- | --- | --- | --- | --- | --- |

- 8.6.2 Deactivate CPU Command

Command

| “C” | 51H | 31H |
| --- | --- | --- |

Positive response

| “P” | 51H | 31H | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 51H | 31H | e1 | e0 |
| --- | --- | --- | --- | --- |

This deactivates CPU card.

- 8.6.3 Inquire CPU Card Status:

Command

| “C” | 51H | 32H |
| --- | --- | --- |

Positive response

| “P” | 51H | 32H | st0 | st1 | st2 | Sti |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 51H | 32H | e1 | e0 |
| --- | --- | --- | --- | --- |

ICRW tells the status of IC card with sti.

St i=30H Card not activated

=31H Card have activated, current CPU Card working frequency is 3.57 MHZ

=32H Card have activated, current CPU Card working frequency is 7.16 MHZ

If ICCard power error, return e1,e0=“60”.

- 8.6.4 CPU Card Communication T=0

Command

| “C” | 51H | 33H | C-APDU |
| --- | --- | --- | --- |

Positive response

| “P” | 51H | 33H | st0 | st1 | st2 | R-APDU |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 51H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

This exchanges data between CPU card by protocol T=0

C-APDU from HOST is range from 4 byte to 261 byte

| CLA | INS | P1 | P2 | LC | Data1 | …… | Le |
| --- | --- | --- | --- | --- | --- | --- | --- |

R-APDU to HOST is range from 2 byte to 258 byte

| Data1 | ….. | Data(n) | Sw1 | Sw0 |
| --- | --- | --- | --- | --- |

An error “60” is returned when a power failure is detected.

If protocol type of IC card is not T=0, error code “62”is sent.

If ICC does not respond within Working Wait Time, CRT-571 deactivates an IC card and error code “63” is sent.

If any other protocol error occurs, CRT-571 deactivates an IC card and error code “64” is sent.

If HOST tries to communicate before IC card activation, error code “65” is sent.

Note: If you want to more about T=0 APDU format. Plese refer to ISO/IEC7816-3 and COS command

- 8.6.5 CPU Card Communication T=1

Command

| “C” | 51H | 34H | C-APDU |
| --- | --- | --- | --- |

Positive response

| “P” | 51H | 34H | st0 | st1 | st2 | R-APDU |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 51H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

This exchanges data between CPU card by protocol T=1

CRT-571 should follow T=1 protocol to combinate C-APDU as I-block and send it to CPU card.CPU card should return R-APDU to HOST

| C-APDU CLA | INS | P1 | P2 | Lc | Data1 | … | Data(Lc) | Le |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| NAD | PCB | LEN | CLA | INS | P1 | P2 | Lc | Data1 | … | Data(Lc) | Le | EDC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Head block |  |  | Information block |  |  |  |  |  |  |  |  | End block |

I-block

CRT-571 returns “R-APDU”data to HOST

| Head block |  |  | Information block |  |  |  |  |  |  |  |  | End block |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAD | PCB | LEN | CLA | INS | P1 | P2 | Lc | Data1 | … | Data(Lc) | Le | EDC |

I-block

| R-APDU CLA | INS | P1 | P2 | Lc | Data1 | … | Data(Lc) | Le |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

An error “60” is returned when a power failure is detected.

If protocol type of IC card is not T=0, error code “62”is sent.

If ICC does not respond within Working Wait Time, CRT-571 deactivates an IC card and error code “63” is sent.

If any other protocol error occurs, CRT-571 deactivates an IC card and error code “64” is sent.

If HOST tries to communicate before IC card activation, error code “65” is sent.

Note: If you want to more about T=0 APDU format. Plese refer to ISO/IEC7816-3 and COS command

    - CPU Warm Reset

Command

| “C” | 51H | 38H |
| --- | --- | --- |

Positive response

| “P” | 51H | 38H | st0 | st1 | st2 | Type | ATR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 51H | 38H | e1 | e0 |
| --- | --- | --- | --- | --- |

Keeping the status of the IC contact activated, returns response upon receiving “ATR” again.

Type: CPU Card communication protocol

=30H         T=0 Protocol

=31H         T=1 Protocol

    - T=1, T=0 CPU Card Protocol Automatic Communication

Command

| “C” | 51H | 39H | C-APDU |
| --- | --- | --- | --- |

Positive response

| “P” | 51H | 39H | st0 | st1 | st2 | R-APDU |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 51H | 39H | e1 | e0 |
| --- | --- | --- | --- | --- |

Protocol is recognized automatically. Set data to “C-APDU”. CRT-571 returns “R-APDU” data to HOST.

An error “60” is returned when a power failure is detected.

If protocol type of IC card is not T=0, error code “62”is sent.

If ICC does not respond within Working Wait Time, CRT-571 deactivates an IC card and error code “63” is sent.

If any other protocol error occurs, CRT-571 deactivates an IC card and error code “64” is sent.

If HOST tries to communicate before IC card activation, error code “65” is sent.

  - SAM(Secure Application Module) Control Command

    - Active SAM Command

Command

| “C” | 52H | 30H | Vcc |
| --- | --- | --- | --- |

Positive response

| “P” | 52H | 30H | st0 | st1 | st2 | Type | ATR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 52H | 30H | e1 | e0 | ATR |
| --- | --- | --- | --- | --- | --- |

The CRT-571 supplies power (VCC) and clock (CLK), then reset (RST) release.

Type: SAM protocol type

=30H T=0 protocol

=31H T=1 protocol

ATR(Answer To Reset) format:

| TS | TO | TA1 | TB1 | … | TCK |
| --- | --- | --- | --- | --- | --- |

See details from ISO7816 standard

Vcc=30H: ICRW supplies with +5V to VCC and activates in line with the EMV2000 ver4.0.

Vcc=33H: ICRW supplies with +5V to VCC and activates in line with the ISO/IEC7816-3.

Vcc=35H: ICRW supplies with +3V to VCC and activates in line with the ISO/IEC7816-3.

Incase there is no Vcc, it will have 30H as default value

If ATR is not compliance to EMV, return e1,e0=“69”

Notes: There will be error and return ATR & Type when reset in line with EMV return

When a power failure is recognized while a power supply is supplied to the card, error code "60" is returned.

    - Deactivate SAM Command

Command

| “C” | 52H | 31H |
| --- | --- | --- |

Positive response

| “P” | 52H | 31H | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 52H | 31H | e1 | e0 |
| --- | --- | --- | --- | --- |

This deactivates SAM

    - Inquire SAM Status Command

Command

| “C” | 52H | 32H |
| --- | --- | --- |

Positive response

| “P” | 52H | 32H | st0 | st1 | st2 | Sti | Stj |
| --- | --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 52H | 32H | e1 | e0 |
| --- | --- | --- | --- | --- |

CRT-571 returns the status of SAM with sti. stj

Sti =30H SAM is deactivated

Sti =31H SAM is activated, working frequency is 3.57 MHZ

Sti =32H SAM is activated, working frequency is 7.16 MHZ

Stj =30H First SAM card connector

Stj =31H Second SAM card connector (Optional)

Stj =32H Third SAM card connector (Optional)

Stj =33H Fourth SAM card connector (Optional)

Stj =34H Fifth SAM card connector (Optional)

An error “60” is returned when a power failure is detected.

    - SAM Communication T=0

Command

| “C” | 52H | 33H | C-APDU |
| --- | --- | --- | --- |

Positive response

| “P” | 52H | 33H | st0 | st1 | st2 | R-APDU |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 52H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

This exchanges data between SAM by protocol T=0

If ICCard power error, return e1,e0=“60”

If protocol type of IC card is not T=0, error code “62”is sent.

If ICC does not respond within Working Wait Time, CRT-571 deactivates an IC card and error code “63” is sent.

If any other protocol error occurs, CRT-571 deactivates an IC card and error code “64” is sent.

If HOST tries to communicate before IC card activation, error code “65” is sent.

Note: If you want to more about T=0 APDU format. Plese refer to ISO/IEC7816-3 and COS command

    - SAM Communication T=1

Command

| “C” | 52H | 34H | C-APDU |
| --- | --- | --- | --- |

Positive response

| “P” | 52H | 34H | st0 | st1 | st2 | R-APDU |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 52H | 44H | e1 | e0 |
| --- | --- | --- | --- | --- |

This exchange data between SAM by protocol T=1

If ICCard power error, return e1,e0=“60”

If protocol type of IC card is not T=0, error code “62”is sent.

If ICC does not respond within Working Wait Time, CRT-571 deactivates an IC card and error code “63” is sent.

If any other protocol error occurs, CRT-571 deactivates an IC card and error code “64” is sent.

If HOST tries to communicate before IC card activation, error code “65” is sent.

Note: If you want to more about T=1 APDU format. Plese refer to ISO/IEC7816-3 and COS command

    - SAM Warm Reset

Command

| “C” | 52H | 38H |
| --- | --- | --- |

Positive response

| “P” | 52H | 38H | st0 | st1 | st2 | Type | ATR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 52H | 38H | e1 | e0 |
| --- | --- | --- | --- | --- |

Keeping the status of the SAM activated, then returns response upon receiving.

Type: SAM protocol type

=30H T=0 Protocol

=31H T=1 Protocol

    - Auto-Check SAM Card T=0/T=1 Protocol

Command

| “C” | 52H | 39H | C-APDU |
| --- | --- | --- | --- |

Positive response

| “P” | 52H | 39H | st0 | st1 | st2 | R-APDU |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 52H | 39H | e1 | e0 |
| --- | --- | --- | --- | --- |

If ICCard power error, return e1,e0=“60”

If protocol type of IC card is not T=0, error code “62”is sent.

If ICC does not respond within Working Wait Time, CRT-571 deactivates an IC card and error code “63” is sent.

If any other protocol error occurs, CRT-571 deactivates an IC card and error code “64” is sent.

If HOST tries to communicate before IC card activation, error code “65” is sent.

    - Select SAM

Command

| “C” | 52H | 40H | SAMn |
| --- | --- | --- | --- |

Positive response

| “P” | 52H | 40H | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 52H | 40H | e1 | e0 |
| --- | --- | --- | --- | --- |

HOST can select SAM 1,2,3,4 or 5.

Sel = 30H: SAM 1.

Sel = 31H: SAM 2. (option)

Sel = 32H: SAM 3. (option)

Sel = 33H: SAM 4. (option)

Sel = 34H: SAM 5. (option)

SAM command is effective only in the module selection.

When Initialize command is executed, SAM 1 will be selected.

- 8.8 	 SLE4442/4428 Control

- 8.8.1 SLE4442/4428 Reset

Command

| “C” | 53H | 30H |
| --- | --- | --- |

Positive response

| “P” | 53H | 30H | st0 | st1 | st2 | ATR(4 byte) |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 54H | 30H | e1 | e0 |
| --- | --- | --- | --- | --- |

The CRT-571 supplies power (VCC) and clock (CLK), then reset (RST) release. After reset, return ATR.

ATR:  	SLE4442 Card ATR=“A2H，13H，10H，91H”

SLE4442 Card ATR=“92H，23H，10H，91H”

- 8.8.2 Deactivate SLE4442/4428

Command

| “C” | 53H | 31H |
| --- | --- | --- |

Positive response

| “P” | 53H | 31H | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 31H | e1 | e0 |
| --- | --- | --- | --- | --- |

The CRT-571 stop suppling power (VCC) and clock (CLK), then reset (RST) release.

- 8.8.3 Inquire status of SLE4442/4428

Command

| “C” | 53H | 32H |
| --- | --- | --- |

Positive response

| “P” | 53H | 32H | st0 | st1 | st2 | Sti |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 54H | 32H | e1 | e0 |
| --- | --- | --- | --- | --- |

CRT-571 tells the status of SLE4442/4428 with Sti after the command successfully execute.

Sti= 30H 	SLE4442/4428   Deactivated

Sti= 31H 	SLE4442        Activated

Sti= 32H  	SLE4428        Activated

- 8.8.4 SLE4442 Control

These functions are specified by a command data form like C-APDU which format is based on T=0 standard.

In this case, CRT-571 recognizes the meaning of the command data, and executes the treatment related to the card by controlling hardware.

After the command was executed properly, CRT-571 returns a positive response with response data 9000H like from the IC card. When an error occurs during the communication with SLE4442, CRT-571 returns a positive response with status information in response data "sw1+sw2” which is base on ISO/IEC 7816-3

| Sw1 | Sw2 | Specification |
| --- | --- | --- |
| 90H | 00H | Success |
| 6FH | 00H | Fail |
| 6FH | 01H | Key Validation error |
| 6FH | 02H | Key Validation error and Lock |
| 67H | 00H | Address overflow |
| 6BH | 00H | Operation length overflow |

- 8.8.4.1. Data read from main memory on SLE4442

Command

| “C” | 53H | 33H | 00H | B0H | 00H | abH | cdH |
| --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 33H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:   ab H: the start address to read data in the main memory

cd H: the length of bytes of data to read

CRT-571 reads data from the main memory of SLE4442, and transmits data on cdH bytes from the address abH.

The capacity of the main memory is 256 bytes.

All the contents of the main memory can be read with the following command.

ex). "CR3"+00B0000000

- 8.8.4.2. Data read from protection memory on SLE4442

Command

| “C” | 53H | 33H | 00H | B0H | 01H | abH | cdH |
| --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 33H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:   ab H: the start address to read data in the main memory

cd H: the length of bytes of data to read

CRT-571 handles the data of all 32bits in the protection memory as the data on 4bytes.

The contents (32bit) of the protection memory can be read with the following command.

Ex)  "CR3"+00B0010004

- 8.8.4.3 Data read from security memory on SLE4442

Command

| “C” | 53H | 33H | 00H | B0H | 02H | abH | cdH | efH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 33H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:   ab H: the start address to read data in the main memory

cd H : the length of bytes of data to read

CRT-571 handles the data of all 32bits in the security memory as the data on 4bytes.

The contents (32bit) of the security memory can be read with the following command.

Ex)  "CR3"+00B0020004

- 8.8.4.4 Data write to main memory on SLE4442

Command

| “C” | 53H | 33H | 00H | D0H | 00H | abH | cdH | efH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 33H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:     ab H: the start address to write data in the main memory

cd H : the length of bytes of data to write

ef H : the data to write first (cd H bytes)

Before write to main memory, the validation of key is must..

The capacity of the main memory is 256 bytes. The byte number "00" of data to write means 256bytes.

The example that data are written in the whole area of the main memory is shown in the following.

ex). "CR3"+ 00D0000000 + Write Data (256byte)

After command execution, CRT-571 returns response with 9000H or sw1+sw2 as the result.

If the addressed data on main memory is protected by the protect status, Data is not allow.

- 8.8.4.5 	Data write to protection memory on SLE4442

Command

| “C” | 53H | 33H | 00H | D0H | 01H | abH | cdH | efH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 33H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:     ab H: the start address to write data in the main memory

cd H : the length of bytes of data to write

ef H : the data to write first (cd H bytes)

Before write to the memory, the validation of key is must..

The address of the main memory that the protection is possible is 1Fh from 00h. Each protection condition of the protectable main memory can be controlled with 4byte (32bits) in the protection memory. For example, if bit0 of the protection memory byte0 is '1', data on the address 00H of the main memory are protected.

The content of protect status can not be change once setting protection.

For exampe: write 20H data to 10H address and set up protection

Ex)   “CR3”+00D001100120

After command execution, CRT-571 returns with 9000H or sw1+sw2 as the result.

ICRW reads data first from the main memory, and it is compared with the value that it was received.

When this is wrong, writing isn't begun.

Protection condition can be set up at one time in the data which continued in the main memory.

- 8.8.4.5 	Data write to security memory on SLE4442

Command

| “C” | 53H | 33H | 00H | D0H | 02H | abH | cdH | efH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 33H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:     ab H: the start address to write data in the main memory

cd H: the length of bytes of data to write

ef H: the data to write first (cd H bytes)

After a password check is finished normally, the Reference-Data area of 3byte can be changed.

All 32bits are handled as 4bytes. How to change the Reference-Data is as the following.

ex). "CR3"+ 00D0020103123456

After command execution, ICRW returns response with 9000H or sw1+sw2 as the result.

Notes: Better not ot writ, because the Error-counter is always allowed to write and easily make

a failure. Error-Counter is controlled when password is checked.

- 8.8.4.6 	Verification data present to SLE4428

Command

| “C” | 53H | 33H | 00H | 20H | 03H | 01H | 03H | efH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 33H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:	ef H: the data to compare (3bytes)

Before changing data, password must be check

Because this function should be made effective, the issue of the next command is necessary.

Ex)  “CR3”+0020030103xxxxxx		(xxxxxx: security code 3bytes)

Card will verify password between card and command.

A user must know password at least when a user wants to rewrite the data on SLE4442 card. Error-Counter can be reset in the zero if password is given to SLE4442 card properly if the value of Error-Counter is 2 or less.

- 8.8.5 SLE4428 Control

These functions are specified by a command data form like C-APDU which format is based on T=0 standard.

In this case, CRT-571 recognizes the meaning of the command data, and executes the treatment related to the card by controlling hardware.

After the command was executed properly, CRT-571 returns a positive response with response data 9000H like from the IC card. When an error occurs during the communication with SLE4442, CRT-571 returns a positive response with status information in response data "sw1+sw2” which is base on ISO/IEC 7816-3

| Sw1 | Sw2 | Specification |
| --- | --- | --- |
| 90H | 00H | Success |
| 6FH | 00H | Fail |
| 6FH | 01H | Key Validation error |
| 6FH | 02H | Key Validation error and Lock |
| 6BH | 00H | Address overflow |
| 67H | 00H | Operation length overflow |

- 8.8.5.1 Data Reading of main-memory of SLE4428

Command

| “C” | 53H | 34H | 00H | B0H | 0aH | bcH | deH |
| --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 34H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:   abc H: the start address to read data in the main memory

de H: the number of bytes of data to read

CRT-571 read data from main memory of SLE4428 through abcH and deH

The capacity of the main memory is 1024bytes.

De="00"

Data to read means 256bytes.

The head part of the main memory can be read with the following command.

Ex)  "CR4"+00B0000000

- 8.8.5.2. Reading of protection-bit of SLE4428

Command

| “C” | 53H | 34H | 00H | B0H | 10H | abH | cdH |
| --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 34H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:  ab H: the start address to read the image of protection data of the main memory

cd H : the number of bytes of data to read

The protection conditions of 1024bytes of main-memory are changed into the data on 1024bits, and it is read.

1024bits is equivalent to 128bytes. (1024 = 128 x 8)

Data to read first become protection information to address (000H-007H) of main-memory in the case of abH=00H.

The contents of the whole protection image can be read with the following command.

ex). "CR4"+00B0100080

CRT-571 read protection-bit of SLE4428 according to abH

- 8.8.5.3 Data writing to main-memory of SLE4428

Command

| “C” | 53H | 34H | 00H | D0H | 0aH | bcH | deH | fgH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 34H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:  abc H: the start address to write data in the main memory

de H: the number of bytes of data to write

fg H: the data to write first (de H bytes)

CRT-571 writes data in the main memory. CRT-571 returns a result after written data are checked.

Before doing this operation, password check must be done

The capacity of the main memory is 1024 bytes.

The example that data are written in from the address 100H is shown in the following.

ex). "CR4"+ 00D0010000 + Write Data (256byte)

After command execution, ICRW returns response with 9000H or sw1+sw2 as the result.

If the addressed data on main memory is protected, the write operation is not available.

- 8.8.5.4 Data writing to main-memory of SLE4428 with protecting

Command

| “C” | 53H | 34H | 00H | D0H | 1aH | bcH | deH | fgH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 34H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:  abc H: the start address to write data in the main memory

de H: the number of bytes of data to write

fg H: the data to write first (de H bytes)

CRT-571 writes data in the main memory. CRT-571 returns a result after written data are checked.

Before doing this operation, password check must be done

- 8.8.5.5 Written with protection-bit

Command

| “C” | 53H | 34H | 00H | D0H | 2aH | bcH | deH | fgH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 34H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:  abc H: the start address to write data in the main memory

de H: the number of bytes of data to write

fg H: the data to write first (de H bytes)

Before doing this operation that writing data with protection-bit, password check must be done

After command execution, ICRW returns response with 9000H or sw1+sw2 as the result.

CRT-571 reads data first from the main memory, and it is compared with the value that it was received.

When this is wrong, writing isn't begun. Protection condition can be set up at a time in the data which continued in the main memory.

- 8.8.5.6 Verification of password present to SLE4428

Command

| “C” | 53H | 34H | 00H | 20H | 00H | 00H | 02H | efH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 53H | 34H | st0 | st1 | st2 | data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 53H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

Notes:	ef H: the data to compare (2bytes)

Before changing data, Password must be checked properly with SLE4428.

Because this function should be made effective, the issue of the next command is necessary.

Ex)  "CR4"+ 0020000002xxxx (xxxx: security code 2bytes)

The presented data are compared with internal data in SLE4428 card itself.

User should know the password of cad if they want to change the data in SLE4442, Error-Counter can be reset in the zero from 7 or less than 7. When error-counter is reset as zero, lock the card.

- 8.9 I2C Memory Card Control Command

- 8.9.1 Activate I2C memory card

Command

| “C” | 54H | 30H | Wrd | Vcc |
| --- | --- | --- | --- | --- |

Positive response

| “P” | 54H | 30H | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 54H | 30H | e1 | e0 |
| --- | --- | --- | --- | --- |

To activate (24C01, 24C02, 24C04, 24C08, 24C16, 24C32, 24C64, 24C128, 24C256) card

CRT-571 supplies a power supply (Vcc), clock (CLK), reset (RST).

Including:

Wrd set I2C type

Wrd =30H 	To activate(24C01,24C02,24C04,24C08,24C16,24C32,24C64,24C128,24C256) card

Wrd =31H   activate 24C01card

Wrd =32H 	 activate 24C02 card

Wrd =33H 	 activate 24C04 card

Wrd =34H 	 activate 24C08 card

Wrd =35H 	 activate 24C16 card

Wrd =36H 	 activate 24C32 card

Wrd =37H 	 activate 24C64 card

Wrd =38H 	 activate 24C128 card

Wrd =39H 	 activate 24C256 card

Vcc choose voltage to card

Vcc=30H	5V

Vcc=31H    3V

Vcc is optional parameter, no Set parameter in command is equal to Set=30H

- 8.9.2 Deactivate I2C memory card

Command

| “C” | 54H | 31H |
| --- | --- | --- |

Positive response

| “P” | 54H | 31H | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 54H | 31H | e1 | e0 |
| --- | --- | --- | --- | --- |

CRT-571 stop suppling a power supply (Vcc), Clock(CLK), Reset(RST).

- 8.9.3 Inquire Status of I2C memory card

Command

| “C” | 54H | 32H |
| --- | --- | --- |

Positive response

| “P” | 54H | 32H | st0 | st1 | st2 | Sti |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 54H | 32H | e1 | e0 |
| --- | --- | --- | --- | --- |

This command is used to inquire status of I2C card and return status by Sti.

Sti meanings:

Sti=30 H 	No I2C be activated

Sti=31 H 	Activated 24C01

Sti=32 H 	Activated 24C02

Sti=33 H 	Activated 24C04

Sti=34 H 	Activated 24C08

Sti=35 H 	Activated 24C16

Sti=36 H 	Activated 24C32

Sti=37 H 	Activated 24C64

Sti=38 H 	Activated 24C128

Sti=39 H 	Activated 24C256

- 8.9.4 I2C Control

These functions are specified by a command data form like C-APDU which format is based on T=0 standard.

In this case, CRT-571 recognizes the meaning of the command data, and execute the treatment related to the card by controlling hardware.

After the command was executed properly, CRT-571 returns a positive response with response data 9000H like from the IC card. When an error occurs during the communication with I2C, CRT-571 returns a positive response with status information in response data "sw1+sw2” which is base on ISO/IEC 7816-3

| Sw1 | Sw2 | Specification |
| --- | --- | --- |
| 90H | 00H | Success |
| 6FH | 00H | Fail |
| 6BH | 00H | Address overflow |
| 67H | 00H | Operation length overflow |

Write/Read I2C and Address scope is showed below:

| Card_type | ab,cd |
| --- | --- |
| 24C01 | 0000H ~ 007FH |
| 24C02 | 0000H ~ 00FFH |
| 24C04 | 0000H ~ 01FFH |
| 24C08 | 0000H ~ 03FFH |
| 24C16 | 0000H ~ 07FFH |
| 24C32 | 0000H ~ 0FFFH |
| 24C64 | 0000H ~ 1FFFH |
| 24C128 | 0000H ~ 3FFFH |
| 24C256 | 0000H ~ 7FFFH |

- 8.9.4.1 Read data from I2C

Command

| “C” | 54H | 33H | 00H | B0H | abH | cdH | efH |
| --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 54H | 33H | st0 | st1 | st2 | Data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 54H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Value:

ab H : The upper address of head address which begins to read data

cd H : The lower address of head address which begins to read data

ef H : The number of bytes of data to read

CRT-571 read efH length and return to HOST according to address specified by abH, cdH.The

length of efH can not be surpass the length of I2C address up limit.

When the following command is transmitted, data can be read from the I2C memory card.

Ex)  "CU3"+00B000000

- 8.9.4.2 Write data to I2C

Command

| “C” | 54H | 34H | 00H | D0H | abH | cdH | efH | ghH… |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 54H | 34H | st0 | st1 | st2 | Data |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 54H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

This command is recognized as follows.

ab H : The upper address of head address which begins to write data

cd H : The lower address of head address which begins to write data

ef H : The number of bytes of data to write

gh H : the data to write first (the head data of the data on ef H bytes)

CRT-571 read efH length and return to HOST according to address specified by abH, cdH.The

length of efH can not be surpass the length of I2C address up limit.

The example which data on 8bytes are written into I2C

Ex)  "CU3"+ 00D0000008 + Write Data (8bytes)

After command execution, ICRW returns response with 9000H or sw1+sw2 as the result.

- 8.10 Contactless IC card Operation

- 8.10.1 Activated contackless IC card

Command

| “C” | 60H | 30H | Set1 | Set2 |
| --- | --- | --- | --- | --- |

(1) Mifare One Card Positive Response

| “P” | 60H | 30H | st0 | st1 | st2 | Rtype | ATQA | UID_len | UID_data | SAK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Mifare One Dard Negative Response

| “N” | 60H | 30H | e1 | e0 | Rtype | ATQA | UID_len | UID_data | SAK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(2) 14443 Type A Card Positive Response

| “P” | 60H | 30H | st0 | st1 | st2 | Rtype | ATQA | UID_len | UID_data | SAK | ATS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

14443 Type A Card Negative Response

| “N” | 60H | 30H | e1 | e0 | Rtype | ATQA | UID_len | UID_data | SAK | ATS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(3) 14443 Type B Card Positive Response

| “P” | 60H | 30H | st0 | st1 | st2 | Rtype | ATQB |
| --- | --- | --- | --- | --- | --- | --- | --- |

14443 Type b Card Negative Response

| “N” | 60H | 30H | e1 | e0 | Rtype | ATQB |
| --- | --- | --- | --- | --- | --- | --- |

Activate RFID card

CRT-571 support activated IEC/ISO14443 Type A and IEC/ISO 14443 Type B

The process is show as below:

1).Mifare one card:		 	1.	Request A ( REQ A)	/ Answer Request A (ATQ A).

2.	Anticollision

3.	Select (SEL) / Unique Identifier (UID) & Select

Acknowledge (SAK)

When Mifare card successfully activate, CRT-571return:

ATQA( 2 byte), UID_data (4—10 byte) and SAK( 1 byte).

2).ISO/IEC 14443 Type A: 		1.	Request A (REQ A)	/ Answer Request A (ATQ A).

2.	Anticollision

3.	Select (SEL) / Unique Identifier (UID) & Select

Acknowledge (SAK)

- Request for answer to select (RATS) / Answer to

Select (ATS)

5.  Protocol and parameter selection request (PPSR) / PPS start(PPSS)

When ISO/IEC 14443 Type A card successfully activated, CRT-571 return:

Mifare card return value increase (ATS (1-254 byte) and protocol parameter (1 byte))

3).ISO/IEC 14443 Type B:		1.   Request B( REQ B)	/ Answer Request B (ATQ B).

2.	Attribute (A TTRIB)	/ Answer to ATTRIB

When ISO/IEC 14443 Type B card successfully activated, CRT-571 return ATQB 12 byte(including following information):

50H, PUPI (4 byte) , App.data(4 byte), Protoclol info(3 byte)

Notes:

Set1, Set2 set sequence of operation for different type of protocol

Valid value: 41H (‘A’= Type A )，42H(‘B’= Type B ), 30H( ‘0’= Do not use)

Ex1：Set1=‘A’，Set2 =‘B’ (default)

Activate sequence： Type A protocol (first sequence), Type B protocol (second sequence)

Ex2：Set1=‘B’，Set2 =‘A’

Activate sequence： Type B protocol (first sequence), Type A protocol (second sequence)

Ex3：Set1=‘A’，Set2 =‘0’

Activate sequence： Type A protocol (first sequence), Type B protocol (Deactivated)

Ex4：Set1=‘B’，Set2 =‘0’，

Activate sequence： Type B protocol (first sequence), Type A protocol (Deactivated)

Rtype:  Protocol

= 41H (‘A’)   In line with ISO/IEC 14443 Type A protocol

= 42H (‘B’)    In line with ISO/IEC 14443 Type B protocol

= 4DH (‘M’)   In line with Philps Mifare one card protocol

When Rtype= 4DH (‘M’)

ATQA= 0044H   Mifare Ultralight Card

ATQA= 0004H   Mifare S50 1K Card

ATQA= 0002H   Mifare S70 4K Card

Mifare one, ISO/IEC 14443 Type A return UID (The length of UID_data)

UID_len=4    The length of UID_data is 4 byte.

UID_len=7    The length of UID_data is 7 byte.

UID_len=10   The length of UID_data is10 byte.

    - Deactivate RFID card

Command

| “C” | 60H | 31H |
| --- | --- | --- |

Positive response

| “P” | 60H | 31H | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 31H | e1 | e0 |
| --- | --- | --- | --- | --- |

Deactivate RFIN card and Output signal to antanna is closed.

    - Inquire status of RFID card

Command

| “C” | 60H | 32H |
| --- | --- | --- |

Positive response

| “P” | 60H | 32H | st0 | st1 | st2 | sti | stj |
| --- | --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 32H | e1 | e0 |
| --- | --- | --- | --- | --- |

Inquire status of RFID sti,stj：

| sti | stj | Specification |
| --- | --- | --- |
| ‘0’ | ‘0’ | Deactivated RF |
| ‘1’ | ‘0’ | Mifare one S50 card |
|  | ‘1’ | Mifare one S70 card |
|  | ‘2’ | Mifare one UL card |
| ‘2’ | ‘0’ | Type A CPU card |
| ‘3’ | ‘0’ | Type B CPU card |

    - Mifare 1 card control

These functions are specified by a command data form like C-APDU which format is based on T=0 standard.

In this case, CRT-571 recognizes the meaning of the command data, and executes the treatment related to the card by controlling hardware.

After the command was executed properly, CRT-571 returns a positive response with response data 9000H like from the IC card. When an error occurs during the communication with Mifare 1 card CRT-571 returns a positive response with status information in response data "sw1+sw2” which is base on ISO/IEC 7816-3.

| Sw1 | Sw2 | Specification |
| --- | --- | --- |
| 90H | 00H | Success |
| 6FH | 00H | Fail |
| 6BH | 00H | Address overflow |
| 67H | 00H | Operation length overflow |

    - Key verification

Command

| “C” | 60H | 33H | 00H | 20H | ks | sn | lc | pdata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Download key to CRT-571 and verify the key directly

ks(1byte):		key select		     (Key A=00H，Key B=01H)

sn(1byte):		sector number 		(S50 card sn=00H-0FH,  S70 card sn=00H-27H)

lc(1byte):		password length lc=06H

pdata(6 byte): 	password data

rdata(2 byte):  	return data( positive response with data 9000H, and negtive response with “ sw1+sw2”)

- 8.10.4.2 Verify key from EEPROM

Command

| “C” | 60H | 33H | 00H | 21H | ks | sn |
| --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Read key from EEPROM of RF module and verify the sector key

Download key via command mentioned in 9.10.4.4

EEPROM can preserve 32 groups of key data

ks(1byte):		key select	              	(Key A=00H，Key B=01H)

sn (1byte):		sector number    			(sn=00H-0FH)

rdata (2 byte):  	return data	( positive response with 9000H)

- 8.10.4.3 Modify sector key (KEY A)

Command

| “C” | 60H | 33H | 00H | D5H | 00H | sn | lc | pdata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Modify sector key (key A)

This command only can modify KEY A, an d modify KEY B as“0xFF, 0xFF, 0xFF,0xFF,0xFF,0xFF in the mean timemodify control words as“0xFF, 0x07, 0x80, 0x69” (ex-work default)

Use block command to modify Key A, Key B control word

sn (1byte):		sector number 			(S50 card sn=00H-0FH,  S70 card sn=00H-27H)

lc (1byte):		password length lc=06H

pdata:    		password data  6 byte.

rdata (2 byte):  	return data

(positive response with data 9000H, and negtive response with “ sw1+sw2”)

- 8.10.4.4	 Download password to EEPROM

Command

| “C” | 60H | 33H | 00H | D0H | ks | sn | lc | pdata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Read key from EEPROM of RF module and verify the sector key

EEPROM can preserve 32 groups of key data

ks(1byte):		key select		    (Key A=00H， Key B=01H)

sn (1byte):		sector number		(sn=00H-0FH)

lc(1byte):		password length      lc=06H

pdata (6 byte): 	password data

rdata (2 byte):	    return data	.

positive response  sw1+sw2=9000H.

negtive response   sw1+sw2=6F00H

- 8.10.4.4 Read sector data

Command

| “C” | 60H | 33H | 00H | B0H | sn | bn | le |
| --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Read block and sequence blocks from RF card

sn (1 byte):		sector number

bn (1 byte):		block number

le (1 byte):		block number  	(le=01H read one block, le=03H read three blocks)

rdata (2 byte):	return data

(Positive response with data 9000H, and negtive response with “ sw1+sw2”)

Notes：

1.Ultralight Card only have one block in one sector，every block have 4 byte data. S50,S70 have16 byte data in one block.

2. Ultralight Card, Mifare 1k (S50), Mifare 1k (S70) card range of capacity is shown as below:

Ultralight Card: sn =00H-0FH,	bn=00H,	le=01H-0FH

Mifare 1k (S50):	sn =00H-0FH,	bn=00H-03H,	le=01H-04H

Mifare 1k (S70):	sn =00H-20H,	bn=00H-03H,	le=01H-04H

sn =21H-27H, bn=00H-0FH,le=01H-10H (S70 card last 8 sector have 16 blocks)

- 8.10.4.5 Write sector data

Command

| “C” | 60H | 33H | 00H | D1H | sn | bn | lc | wdata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Read block and sequence blocks from RF card

sn (1 byte):		sector number

bn (1 byte):		block number

le (1 byte):		block number

wdata:		    block to write (n byte)

rdata (2 byte):	    return data

(Positive response with data 9000H and negtive response with “sw1+sw2”)

Notes：

1. Ultralight Card only have one block in one sector，every block have 4 byte data. S50,S70 have16 byte data in one block

2. Ultralight Card, Mifare 1k(S50), Mifare 1k (S70) card card range of capacity is shown as below:

Ultralight Card:  sn=00H-0FH,	bn=00H-03H,	lc=01H-03H

Mifare 1k(S50):	 sn=00H-0FH,	bn=00H-03H,	lc=01H-03H

Mifare 1k(S70):	 sn=00H-20H,	bn=00H-03H,	lc=01H-03H

sn=21H-27H,	bn=00H-0FH,	lc=01H-0FH

(S70 card last 8 sector have 16 blocks)

3. S50,S70 card last block of each sector is control sector to preserve Key A, read/write control words, Key B.

Cautions: Do note write last block and CRT-571 also will prohibid to write last block.

- 8.10.4.6 Initialization

Command

| “C” | 60H | 33H | 00H | D2H | sn | bn | lc | wdata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Initialization operation to RF card

sn(1 byte):		sector number

bn(1 byte):		block number

lc(1byte):		length  lc=04H

wdata:		    data (4 byte)

rdata (2 byte):   	return data

(Positive response with data 9000H and negtive response with “sw1+sw2”)

Notes：Mifare 1k(S50), Mifare 1k (S70) card operation sector

(Sector can not be out of range and last block can not be operated)

Mifare 1k (S50):	 sn=00H-0FH,	bn=00H-03H,

Mifare 1k (S70):	 sn=00H-20H,	bn=00H-03H,

sn=20H-27H,	bn=00H-0EH,

(S70 card last 8 sector have 16 blocks)

- 8.10.4.7 Read value

Command

| “C” | 60H | 33H | 00H | B1H | sn | bn |
| --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Read value operations to RF card

sn (1 byte):		sector number

bn (1 byte):		block number

rdata (2 byte):  return data

(Positive response with data 9000H and negtive response with “sw1+sw2”)

Notes：Mifare 1k (S50), Mifare 1k (S70) card operation sector

(Sector can not be out of range and last block can not be operated)

Mifare 1k (S50):	 sn=00H-0FH,	bn=00H-03H,

Mifare 1k (S70):	 sn=00H-20H,	bn=00H-03H,

sn=20H-27H,	bn=00H-0EH,

(S70 card last 8 sector have 16 blocks)

- 8.10.4.8 Increment

Command

| “C” | 60H | 33H | 00H | D3H | sn | bn | lc | wdata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Increment operation to RF card

sn (1 byte):		sector number

bn (1 byte):		block number

lc (1byte):		increment length lc=04H

wdata:		increment data (4 byte)

rdata(2 byte):  return data

(Positive response with data 9000H, and negtive response with “ sw1+sw2”)

Notes：Mifare 1k (S50), Mifare 1k (S70) card operation sector

(Sector can not be out of range and last block can not be operated)

Mifare 1k (S50):	 sn=00H-0FH,	bn=00H-03H,

Mifare 1k (S70):	 sn=00H-20H,	bn=00H-03H,

sn=20H-27H,	bn=00H-0EH,

(S70 card last 8 sector have 16 blocks)

- 8.10.4.9 Decrement

Command

| “C” | 60H | 33H | 00H | D4H | sn | bn | lc | wdata |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Positive response

| “P” | 60H | 33H | st0 | st1 | st2 | rdata |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 33H | e1 | e0 |
| --- | --- | --- | --- | --- |

Decrement operation to RF sector

sn (1 byte):		sector number

bn (1 byte):		block number

lc (1byte):		Decrement length lc=04H

wdata:		    Decrement data(4 byte)

rdata (2 byte):  return data

(Positive response with data 9000H, and negtive response with “ sw1+sw2”)

Notes：Mifare 1k(S50), Mifare 1k (S70) card operation sector

(Sector can not be out of range and last block can not be operated)

Mifare 1k (S50):	 sn=00H-0FH,	bn=00H-03H,

Mifare 1k (S70):	 sn=00H-20H,	bn=00H-03H,

sn=20H-27H,	bn=00H-0EH,

(S70 card last 8 sector have 16 blocks)

- 8.10.3 Type A RF card communication

Command

| “C” | 60H | 34H | C-APDU |
| --- | --- | --- | --- |

Positive response

| “P” | 60H | 34H | st0 | st1 | st2 | R-APDU |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 34H | e1 | e0 |
| --- | --- | --- | --- | --- |

This exchanges data between RF card by protocol RF Type A T=CL according to ISO/IEC 14443-4

Notes:  The max. length of C-APDU is 261 byte, the max. length of R-APDU is 258 byte.

- 8.10.4 Type B RFcard communication

Command

| “C” | 60H | 35H | C-APDU |
| --- | --- | --- | --- |

Positive response

| “P” | 60H | 35H | st0 | st1 | st2 | R-APDU |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | 60H | 35H | e1 | e0 |
| --- | --- | --- | --- | --- |

This exchanges data between RF card by protocol RF Type B T=CL according to ISO/IEC 14443-4

Notes: The max. length of C-APDU is 261 byte, the max. length of R-APDU is 258 byte.

- 8.11 Read Serial Number of CRT-571

- 8.11.1 Read serial number

Command

| “C” | A2H | 30H |
| --- | --- | --- |

Positive response

| “P” | A2H | 30H | st0 | st1 | st2 | len | ICRW_SN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | A2H | 30H | e1 | e0 |
| --- | --- | --- | --- | --- |

Len:   read length of CRT-571serial number (0byte-18byte)

ICRW_SN:  CRT-571 serial number

- 8.11.2 Write Serial Number of CRT-571

- Omitted

- 8.11.3 Read CRT-571 configuration

Command

| “C” | A3H | 30H |
| --- | --- | --- |

Positive response

| “P” | A3H | 30H | st0 | st1 | st2 | ICRW_Config |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | A3H | 30H | e1 | e0 |
| --- | --- | --- | --- | --- |

CRT-571 configuration specification：

| Name | Value | Description |
| --- | --- | --- |
| S1 |  | CRT Reader Identifier word |
|  | “7” | S1 =“37” |
| S2/S3/S4 (3 Byte) |  | User Code option |
|  | “V10” | CRT version |
|  | “XXX” | Customize version |
| S5 |  | Card r/w type option |
|  | “0” | Dispensing available, Read/Write unavailable |
|  | “I” | IC card r/w |
|  | “C” | RF card r/w |
|  | “E” | IC + RF card r/w |
| S6 |  | Interface type option |
|  | “R” | RS-232Interface type |
| S7 |  | IC card write type |
|  | “0” | IC card writing unavailable |
|  | “1” | IC card connector for third-party usage |
|  | “2” | Standard IC card read/write |
| S8 |  | RF card write type |
|  | “0” | RF card write/read unavailable |
|  | “1” | RF card antenna for third-party usage |
|  | “2” | Standard RF card read/write |
| S9 |  | SAM option |
|  | “0” | Not SAM |
|  | “1” | SAM 1 |
|  | “2” | SAM 2 |
|  | “3” | SAM 3 |
|  | “4” | SAM 4 |
|  | “5” | SAM 5 |
| S10 | “0” | Components related to dispense cards |
|  | “1” | Components related to remove cards |

- 8.11.4 Read CRT-571 version information

Command

| “C” | A4H | Pm |
| --- | --- | --- |

Positive response

| “P” | A4H | 30H | st0 | st1 | st2 | Rev |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | A4H | 30H | e1 | e0 |
| --- | --- | --- | --- | --- |

Read CRT-571 version information

Pm=30H		Read machine software information

Ex:		Rev =“C571_V1.00_A_090910”

Pm=31H		Read IC Card software information

Ex:		Rev =“ICCARD_V10_A_090910”

Pm=32H		Read RF Card software information

Ex:		Rev =“RFCARD_V10_A_090910”

- 8.12.1 Error-card Bin Counter Control

- 8.13.1 Read error-card bin counter

Command

| “C” | A5H | 30H |
| --- | --- | --- |

Positive response

| “P” | A5H | 30H | st0 | st1 | st2 | Count(3 byte) |
| --- | --- | --- | --- | --- | --- | --- |

Negative response

| “N” | A5H | 30H | e1 | e0 |
| --- | --- | --- | --- | --- |

After reset error-card bin counter, Capture on card, counter one plus

Count= “000” ~ “999”

Counter overflow will return machine status (e1,e0=“50”)

- 8.13.2 Set initial value of error-card bin

Command

| “C” | A5H | 31H | Count(3 byte) |
| --- | --- | --- | --- |

Positive response

| “P” | A5H | 31H | st0 | st1 | st2 |
| --- | --- | --- | --- | --- | --- |

Negative response

| “N” | A5H | 31H | e1 | e0 |
| --- | --- | --- | --- | --- |

Set initial value of error-card bin.

Count= “000” ~ “999”

Count value range (0-999)